# Real Time Operating Systems Design and Programming

LAB 1

OS Assembly Lab
Processing Text in Assembly Language

- Contents

1	Overview	1

2	Software	1

2.1	Mixing Assembly Language and C Code	1

2.2	Main	1

2.3	Register Use Conventions	1

2.3.1	Calling functions and Passing Arguments	2

2.3.2	Temporary Storage	2

2.3.3	Preserved Registers	2

2.3.4	Returning from Functions	2

2.4	String Copy	2

2.5	String Capitalization	2

3	Lab Procedure	3

# Overview

In this exercise, you will execute assembly code on your board using the debugger in order to examine its execution at the processor level. This lab aims to get you familiar with the development environment and understand how the hardware “plumbing” works.

# Software

## Mixing Assembly Language and C Code

We will use MDK to produce a C program but add assembly language subroutines to perform the string copy and capitalisation operations. Some embedded systems are coded purely in assembly language, but most are coded in C and use assembly language only for time-critical processing. This is because the code development process is much faster (and hence much less expensive) when writing in C compared to assembly language. Writing an assembly language function that can be called as a C function results in a modular program that offers us the best of both worlds: the fast, modular development of C and the high performance of assembly language. It is also possible to add inline assembly code to C code, but this requires a much greater knowledge of how the compiler generates code.

## Main

First, we will create the main C function. This function contains two variables (a and b) with character arrays.

int main(void)

{

const char a[] = "Hello world!";

char b[20];

my_strcpy(a, b);

my_capitalize(b);

while (1)

;

}

## Register Use Conventions

There are certain register use conventions that we need to follow if we want our assembly code to coexist with C code. They are not particularly important to this course, but the point is to show you how an OS developer must be careful about such hardware details.

### Calling Functions and Passing Arguments

When a function calls a subroutine, it places the return address in the link register (lr). The arguments (if any) are passed in registers r0 through r3, starting with r0. If there are more than four arguments, or they are too large to fit in 32-bit registers, they are passed on the stack.

### Temporary Storage

Registers r0 through r3 can be used for temporary storage if they were not used for arguments, or if the argument value is no longer needed.

### Preserved Registers

Registers r4 through r11 must be preserved by a subroutine. If any must be used, they must be saved first and restored before returning. This is typically done by pushing them to and popping them from the stack.

### Returning from Functions

Because the return address has been stored in the link register, the BX lr instruction will reload the PC with the return address value from the lr. If the function returns a value, it will be passed through register r0.

## String Copy

The function `my_strcpy` has two arguments (src, dst). Each is a 32-bit pointer to a character. In this case, a pointer fits into a register, so the argument `src` is passed through register r0 and `dst` is passed through r1.

Our function will load a character from memory.

my_strcpy

loop

LDRB    r2, [r0]       ; Load byte from src

ADDS    r0, r0, #1     ; Increment src pointer

STRB    r2, [r1]       ; Store byte in dst

ADDS    r1, r1, #1     ; Increment dst pointer

CMP     r2, #0         ; Check if null terminator

BNE     loop           ; If not, repeat

BX      lr             ; Return

## String Capitalization

Let’s look at a function to capitalise all the lowercase letters in the string. We need to load each character, check to see if it is a letter, and if so, capitalize it.

Each character in the string is represented by its ASCII code. For example, ‘A’ is represented with a 65 (0x41), ‘B’ with 66 (0x42), and so on up to ‘Z’ which uses 90 (0x5a). The lowercase letters start at ‘a’ (97, or 0x61) and end with ‘z’ (122, or 0x7a). We can convert a lowercase letter to an uppercase letter by subtracting 32.

my_capitalize

cap_loop

LDRB    r1, [r0]       ; Load byte from str

CMP     r1, #'a'-1     ; Compare with 'a' - 1

BLS     cap_skip       ; If lower, skip

CMP     r1, #'z'       ; Compare with 'z'

BHI     cap_skip       ; If higher, skip

SUBS    r1, r1, #32    ; Convert to uppercase

STRB    r1, [r0]       ; Store modified byte

cap_skip

ADDS    r0, r0, #1     ; Increment pointer

CMP     r1, #0         ; Check for null terminator

BNE     cap_loop       ; If not null, continue

BX      lr             ; Return

The code is shown above. It loads the byte into r1. If the byte is less than ‘a’, then the code skips the rest of the tests and proceeds to finish the loop iteration.

This code has a quirk—the first compare instruction compares r1 against the character immediately before ‘a’ in the table. Why? What we would like is to compare r1 against ‘a’ and then branch if it is lower. However, there is no branch lower instruction, only branch lower or same (BLS). To use that instruction, we need to reduce by one the value we compare r1 against.

# Lab Procedure

- Copy the main.c and my_asm.asm files provided to your empty project.

- Compile the code.

- Run the Debugger

- Run the program until the opening brace in the main function is highlighted. Open the Registers window (ViewRegisters Window). What are the values of the stack pointer (r13), link register (r14) and the program counter (r15)?

sp = 0x20000678, lr = 0x080002AF, pc = 0x08001D70.

- Open the Disassembly window (ViewDisassembly Window). Which instruction does the yellow arrow point to, and what is its address? How does this address relate to the value of PC?

PUSH {r7, lr} is at address 0x08001D70, which is the value of PC. This is the next instruction that will be executed.

- Step one machine instruction forward using the F10 key while the Disassembly window is selected. Which two registers have changed (they should be highlighted in the Registers window), and how do they relate to the instruction just executed?

The stack pointer r13 has changed to 0x20000670. The program counter r15 has changed to 0x08001D72. SP changed because it set up the stack frame, and PC changed because it stepped into the next instruction

- Continue execution (using F10) until reaching the BL `my_strcpy` instruction. What are the values of the SP, PC and LR?

SP = 0x20000640, LR = 0x0800053B, PC = 0x08001DAA.

- Execute the BL instruction. What are the values of the SP, PC and LR? What has changed and why? Does the PC value agree with what is shown in the Disassembly window?

SP = 0x20000640, LR = 0x08001DB1, PC = 0x08001DB0.lr has changed because the BL instruction saved the return address (old value of PC + length of BL instruction +1). PC has changed because the PC is loaded with the address of the subroutine to execute. Yes, the PC matches the disassembly window contents – the yellow arrow points to the instruction at 0x08001DB0.

- What registers hold the arguments to `my_strcpy` (src and dst), and what are their contents?

src: register r0, value 0x2000065C

dst: register r1, value 0x20000648

- Open a Memory window (ViewMemory WindowsMemory 1) for the address for src determined above. Open a Memory window (ViewMemory WindowsMemory 2) for the address for dst determined above. Right-click on each of these memory windows and select ASCII to display the contents as ASCII text

- What are the memory contents addressed by src?

Hello world!

- What are the memory contents addressed by dst?

Null characters, displayed as ……………. in ASCII mode.

- Steps through the assembly code two instructions at a time while watching memory window 2 to see the string being copied character by character from src to dst. Which register holds the character?

r0 holding 0x2000648

- What are the values of the character, the src pointer, the dst pointer, the link register (r14) and the program counter (r15) when the code reaches the last instruction in the subroutine (BX lr)?

r0 = 0x20000669, r1 = 0x20000655, LR = 0x08001DB1, PC = 0x08001DB0

- Execute the BX lr instruction. Now what is the value of the PC?

PC = 0x08001DB2

- What is the relationship between the PC value and the previous LR value? Explain.

PC is LR-1. The processor resumes executing code at address 0x08001454 (PC), but the last bit of the LC is set to indicate the processor is executing in Thumb mode. This is due to the fact that the first bit of PC is always set to 0 so instructions are always aligned to word or halfword boundaries.

- Now step through the `my_capitalize` subroutine and verify that it works correctly, converting b from “Hello world!” to “HELLO WORLD!”.

In embedded software, all these do not have to be handled by an OS (or by the compiler); however, the OS usually performs similar housekeeping and makes development abstracted from the low-level hardware details.