# Real Time Operating Systems Design and Programming

Lab 04

OS Scheduler Lab

Evaluating Scheduler Responsiveness

- Contents

1	Overview	2

1.1	Software	2

2	Procedure	4

2.1	Non-pre-emptive non-prioritized	4

2.2	Non-pre-emptive prioritized	5

2.3	Pre-emptive prioritized	6

# Overview

In this lab, you will evaluate how changing the scheduling approach (prioritization and pre-emption) affects how quickly a multithreaded program responds to an input event.

The goal of this program is to drive the LEDs through a sequence of colors slowly (red, blue, green, repeat). When the user presses a switch, the system should quickly flash all LEDs as long as the switch is pressed. The time delay between the user pressing the switch and seeing the LED flash is the system’s response time, which is highly dependent on the type of scheduler used.

## Software

There are four software threads in the system.

- Three threads control the LEDs (one LED per thread). Each of these LED threads lights its LED, executes a busy-wait loop (simulating a long computation), turns off the LED, and yields the processor. In systems with prioritization, the red LED thread has the highest priority, followed by  the blue LED thread, and then finally the green LED thread.

- One thread (PS) polls the blue (user) switch and flashes the LEDs as long as the switch is pressed. When the switch is released, the thread yields the processor. When the LEDs are flashing, this indicates that this thread (PS) is running and has detected the switch has been pressed.

Figure 1. Ideal thread release schedule for PS, R, G and B.

The ideal thread schedule is shown in Figure 1. The following timing behavior is used for the prioritized cases:

- The poll switch (PS) thread is scheduled to run very frequently (about every 1 millisecond) to detect user input.

- The LED threads are scheduled to be released periodically (about every 6 seconds) and light the LED for about a second.

- Initially, all threads are released simultaneously.

The non-pre-emptive, non-prioritized approach runs each thread in a fixed order (PS, R, B, G) as fast as possible.

You will compare how long it takes the system to start flashing the LED after you press the switch using three separate scheduling approaches.

- We will use a superloop for non-pre-emptive non-prioritized scheduling. In this case, a simple infinite loop calls the four thread functions sequentially.

- The RTX kernel can emulate non-pre-emptive prioritized scheduling. This is done by having a thread raise its priority to the maximum level when it starts running, and then restore its original priority upon completion.

- The RTX kernel provides pre-emptive prioritized scheduling. You will evaluate thread response time for two cases: when the switch thread has lower priority than the LED threads, and when it has higher priority.

For convenience, we use a wrapper function around each function to create a thread that the RTOS can manage. We could eliminate the wrapper function, but then we would need to modify the internals of the thread function to switch between schedulers.

# Procedure

Select the scheduling approach by setting one of these symbols to 1: SCHED_NPRE_NPRI, SCHED_NPRE_PRI, SCHED_PRE_PRI.

## Non-pre-emptive non-prioritized

- Open the template project. Scan through the code.

- Configure the code to use non-pre-emptive non-prioritized scheduling (by defining SCHED_NPRE_NPRI as 1, and the others as zero). Build the project and download the executable to the MCU. Press the user switch (blue) as soon as the red LED lights up and observe the response. Complete the table below to show the LED activity before and during pressing the switch.

| Switch | open | closed (pressed) |
| --- | --- | --- |
| LED Activity | red blue green flashing |  |

- Which threads ran between the time when you closed the switch and when the LED started flashing? Did they run for their full duration, or did they stop early (i.e., were they pre-empted)? Explain how the scheduler affected this. What controls the order in which the threads run?

The threads for red, blue and green ran (without pre-emption) and then the PS thread could run. The scheduler uses a fixed schedule without thread pre-emption. The threads ran in the order R, B, G, PS because the superloop calls them in order PS, R, B, G and then repeats. Pressing the switch during the red thread means that PS doesn’t get to run until the next time through the superloop.

- Draw a timeline showing processor activity corresponding to your LED chart.

## Non-pre-emptive prioritized

- Configure the code to use non-pre-emptive prioritized scheduling (SCHED_NPRE_PRI as 1). Configure the `Poll_Switch` thread to have lower priority than the LED threads by defining PS_PRIORITY as 1. Build the project and download the executable to the MCU. Press the switch as soon as the red LED lights and observe the response. Complete the chart below to show the LED activity before and during pressing the switch.

| Switch | open | closed (pressed) |
| --- | --- | --- |
| LED Activity | green red blue green flashing |  |

- Which threads ran between when you closed the switch and when the LED started flashing? Did they run for their full duration, or did they stop early (i.e., were they pre-empted)? Explain how the scheduler affected this. What controls the order in which the threads run?

The threads for red, blue, and green ran (without pre-emption) and then the PS thread could run. The scheduler uses priority to select the thread but doesn’t use thread pre-emption. The threads ran in the order R, B, G, PS because their priorities are in that order, starting with the highest (R) through the lowest (PS). Pressing the switch during the red thread means that PS doesn’t get to run until all threads with a higher priority than PS have run.

- Draw a timeline showing processor activity corresponding to your LED chart.

- Configure the code to use non-pre-emptive prioritized scheduling. Configure the Poll_Switch thread to have higher priority than the LED threads by defining PS_PRIORITY as 5. Build the project and download the executable to the MCU. Press the switch as soon as the red LED lights up and observe the response. Complete the chart below to show the LED activity before and during pressing the switch.

| Switch | open | closed (pressed) |
| --- | --- | --- |
| LED Activity | green red flashing |  |

- Which threads ran between when you closed the switch and when the LED started flashing? Did they run for their full duration, or did they stop early (i.e., were they pre-empted)? Explain how the scheduler affected this. What controls the order in which the threads run?

The thread for red ran (without pre-emption), and then the PS thread could run. The scheduler uses priority to select the thread but doesn’t use thread pre-emption.  Although thread PS has the highest priority, it doesn’t get to run until thread R finishes and the scheduler picks the next thread to execute.

- Draw a timeline showing processor activity corresponding to your LED chart.

N

## Pre-emptive prioritized

- Configure the code to use pre-emptive prioritized scheduling. Configure the Poll_Switch thread to have lower priority than the LED threads by defining PS_PRIORITY as 1. Build the project and download the executable to the MCU. Press the switch as soon as the red LED lights and observe the response. Complete the chart below to show the LED activity before and during pressing the switch.

| Switch | open | closed (pressed) |
| --- | --- | --- |
| LED Activity | off red blue green flashing |  |

- Which threads ran between when you closed the switch and when the LED started flashing? Did they run for their full duration, or did they stop early (i.e., were they pre-empted)? Explain how the scheduler affected this. What controls the order in which the threads run?

The threads for red, blue, and green ran (without being pre-empted) and then the PS thread could run. The scheduler uses priority to select the thread and uses thread pre-emption. Even though we are using pre-emptive scheduling, the PS thread has lower priority than R, G, or B, so the scheduler does not run it until R, G, and B have finished. The threads ran in the order R, B, G, PS because their priorities are in that order, starting with the highest (R) through the lowest (PS). Pressing the switch during the red thread means that PS doesn’t get to run until all threads with a higher priority than PS have run.

- Draw a timeline showing processor activity corresponding to your LED chart.

- Configure the code to use pre-emptive, prioritized scheduling. Configure the Poll_Switch thread to have higher priority than the LED threads by defining PS_PRIORITY as 5. Build the project and download the executable to the MCU. Press the switch as soon as the red LED lights up and observe the response. Complete the chart below to show the LED activity before and during pressing the switch.

| Switch | open | closed (pressed) |
| --- | --- | --- |
| LED Activity | off red flashing |  |

- Which threads ran between when you closed the switch and when the LED started flashing? Did they run for their full duration, or did they stop early (i.e., were they pre-empted)? Explain how the scheduler affected this. What controls the order in which the threads run?

All running threads are now pre-empted every 1 ms by the PS thread. If the switch is not pressed, PS returns very quickly without changing the LED, so we do not see any effect on the system. When the switch is finally pressed, we see the LED flashing immediately. In fact, this is not immediate, but instead a delay of up to 1 ms (the period at which PS runs). For a pre-emptive system to work effectively, we need to allocate the priorities correctly.

- Draw a timeline showing processor activity corresponding to your LED chart.